Udacity Self-Driving Car Nanodegree

Project 4- Advanced Lane Finding

Author: Johannes Betz

Date: 07/21/2017

Table of Contents

1.	Introduction and goals	3

2.	Camera Calibration	3

3.	Pipeline for single Images	4

a.	Distortion Correction	4

b.	Creating a binary Treshold image: Color Transformation, Gradient Transformation	5

c.	Perspective Transformation	8

d.	Finding lane line pixels and fit position with a polynomial	8

e.	Calculating the radius of a curvature and defining the position of the vehicle	8

f.	Example Image	8

4.	Pipeline for a video	9

5.	Discussion	9

# Introduction and goals

The following writeup includes the code and the description for an advance lane finding algorithm.The goals / steps of this project are the following:

- Compute the camera calibration matrix and distortion coefficients given a set of chessboard images.

- Apply a distortion correction to raw images.

- Use color transforms, gradients, etc., to create a thresholded binary image.

- Apply a perspective transform to rectify binary image ("birds-eye view").

- Detect lane pixels and fit to find the lane boundary.

- Determine the curvature of the lane and vehicle position with respect to center.

- Warp the detected lane boundaries back onto the original image.

- Output visual display of the lane boundaries and numerical estimation of lane curvature and vehicle position.

Every section provides one part of the project rubics.

# Camera Calibration

For the camera calibration 20 chessboard pictures are provided in the folder camera_cal. Each chessboard picture can be used for the camera calibration. The code looks like this:

def calibrate_camera(calibration_images, nx, ny):

    # Arrays to store object points and image points from all the images
    realpoints = []  # 3D points in real world space
    imagepoints = []  # 2D points in image plane

    # Prepare object points by creating  6x8 points in an array each with 3 columns for the x,y,z coordinates of each corner
    objp = np.zeros((ny * nx, 3), np.float32)

    # Use numpy mgrid function to generate the coordinates that we want
    objp[:, :2] = np.mgrid[0:nx, 0:ny].T.reshape(-1, 2)

    for name in calibration_images:

        img = mpimg.imread(name)

        gray = cv2.cvtColor(img, cv2.COLOR_RGB2GRAY)

        # Find the chessboard corners
        ret, corners = cv2.findChessboardCorners(gray, (nx, ny), None)

        # If corners are found, add image points and object points
        if (ret):
            imagepoints.append(corners)

            # Object points will be the same for all of the calibration images
            # Since they represent a real chessboard
            realpoints.append(objp)

    return cv2.calibrateCamera(realpoints, imagepoints, img.shape[0:2], None, None)


nx = 9
ny = 6
calibration_images = glob.glob("camera_cal/*")

# Calibrate the camera
ret, mtx, dist, rvecs, tvecs = calibrate_camera(calibration_images, nx, ny)

First, the number of inner corners per row and column are counted (6 and 9). Then, the function calibrate_camera is called to calibrate the camera. First, each pictures is read with the imread() function. Then, the picture is turned into greyscale with the cv2.cvtColor(img, cv2.COLOR_RGB2GRAY) function.

The corners are found by the cv2.findChessboardCorners function, where the grayscaled pictures are integrated. After that, a calibrated image looks like this:

# Pipeline for single Images

The following chapter 3 includes a complete pipeline for including pictures and reading information out o fit. Each step is written in one section.

# Distortion Correction

The function_calibrate camera produces the variable „mtx“. This variable includes the camera coefficient matrix. In addition, the distrotation points are given back. The code for the distoration correction is listed below:

def undistort(image):

    return cv2.undistort(image, mtx, dist)


def plot_on_subplots(images, titles, cmap=None):

    f, (ax1, ax2) = plt.subplots(1, 2, figsize=(20, 10))

    if cmap:
        ax1.imshow(images[0], cmap=cmap)
    else:
        ax1.imshow(images[0])
    ax1.set_title(titles[0])

    if (cmap):
        ax2.imshow(images[1], cmap=cmap)
    else:
        ax2.imshow(images[1])

    ax2.set_title(titles[1])

The distortation correction is done for two test images

Example Image 1

- # Perform distortion Correction on one of the calibration images
calibration_image = plt.imread("camera_cal/calibration1.jpg")

plot_on_subplots([calibration_image, undistort(calibration_image)], ["Original Image", "Distortion Corrected Image"])

Example Image 2

# Perform un-distortion on a test images
test_image = plt.imread("test_images/test5.jpg")
plot_on_subplots([test_image, undistort(test_image)], ["Original Image", "Distortion Corrected Image"])

# Creating a binary Treshold image: Color Transformation, Gradient Transformation

After the pictures are distorted, the goal was to create a binary threshold image. To create a binary threshold image, different steps are executed:

- Apply the sobel operator: converts the image to grayscale, applies a sobel operator in the X direction, takes the absolute value, scales the result in the range 0-255 and performs a thresholding operation.

- HLS Channel binary: performs thresholding on either the H, L or S channel of the image depending on the input parameter.

- Greyscale Treshold: converts to grayscale using the cv2 cvtColor function and performs thresholding using the thresh parameter.

- Color Selection: Performs color selection on the image converted to HLS color space by only selecting Yellow and White colors and returns the binary mask for the same

- Combine: combines the above explained 4 operations to produce a binary image of the input image.

The Code for the Binary Treshold image is listed here:

def get_sobel_binary(image, thresh_min=20, thresh_max=200):

    gray = cv2.cvtColor(image, cv2.COLOR_RGB2GRAY)

    # Take the sobel derivative in the x direction
    sobelx = cv2.Sobel(gray, cv2.CV_64F, 1, 0)

    # Absolute x derivative to accentuate lines away from horizontal
    abs_sobelx = np.absolute(sobelx)

    # Scale to 8 bit grayscale image
    scaled_sobel = np.uint8(255 * abs_sobelx / np.max(abs_sobelx))

    # Apply a threshold
    sxbinary = np.zeros_like(scaled_sobel)
    sxbinary[(scaled_sobel >= thresh_min) & (scaled_sobel <= thresh_max)] = 1

    return sxbinary


def get_channel_index_hls(chanel_name):

    if (chanel_name == "h"):
        return 0
    elif (chanel_name == "l"):
        return 1
    elif (chanel_name == "s"):
        return 2


def get_hls_channel_binary(image, channel_name='s', thresh_min=180, thresh_max=255):

    # Convert to HLS color space
    hls = cv2.cvtColor(image, cv2.COLOR_RGB2HLS)

    # Extract the desired channel
    channel_index = get_channel_index_hls(channel_name)
    channel = hls[:, :, channel_index]

    # Apply the threshold
    channel_binary = np.zeros_like(channel)
    channel_binary[(channel >= thresh_min) & (channel <= thresh_max)] = 1

    return channel_binary


def get_grayscale_thresholded_img(img, thresh=(130, 255)):

    gray = cv2.cvtColor(img, cv2.COLOR_RGB2GRAY)

    binary = np.zeros_like(gray)

    binary[(gray > thresh[0]) & (gray < thresh[1])] = 1

    return binary


def get_color_selection(image):

    hsv = cv2.cvtColor(image, cv2.COLOR_RGB2HSV)

    lower_yellow = np.array([0, 100, 100], dtype=np.uint8)
    upper_yellow = np.array([190, 250, 255], dtype=np.uint8)

    upper_white = np.array([200, 200, 200], dtype=np.uint8)
    lower_white = np.array([255, 255, 255], dtype=np.uint8)

    # Get the white pixels from the original image
    mask_white = cv2.inRange(image, upper_white, lower_white)

    # Get the yellow pixels from the HSV image
    mask_yellow = cv2.inRange(hsv, lower_yellow, upper_yellow)

    # Bitwise-OR white and yellow mask
    mask = cv2.bitwise_or(mask_white, mask_yellow)

    return mask

def get_binary_image(image):


    sobel_binary = get_sobel_binary(image)

    s_channel_binary = get_hls_channel_binary(image)

    l_channel_binary = get_hls_channel_binary(image, channel_name="l", thresh_min=200)

    gray_scale_thresholded_image = get_grayscale_thresholded_img(image)

    color_sel = get_color_selection(image)

    combined_binary = np.zeros_like(s_channel_binary)

    combined_binary[(color_sel == 255) | ((s_channel_binary == 1) & (l_channel_binary == 1))
                    | ((sobel_binary == 1) & (gray_scale_thresholded_image == 1)) | (l_channel_binary == 1)] = 1

    return combined_binary

The complete binary image pipeline is displayed wit the one of the test files and is executed like this:

bin_image = get_binary_image(test_image)
plot_on_subplots([test_image , bin_image], ["Original Image", "Binary Image"], cmap="gray")

# Perspective Transformation

# Finding lane line pixels and fit position with a polynomial

# Calculating the radius of a curvature and defining the position of the vehicle

# Example Image

As the final step, the whole pipeline is exercised with different test files. The whole pipeline for processing one image can be found here:

def process_image(image):


    try:
        blurred_img = gaussian_blur(image)

        undistorted_image = undistort(blurred_img)

        binary_image = get_binary_image(undistorted_image)

        roi_image = select_region_of_interest(binary_image)

        persp_transform = transform_perspective(roi_image, M)

        lane_base = get_lane_lines_base(persp_transform)

        left_base, right_base = lane_base

        left_pixels = get_lane_pixels(persp_transform, left_base)

        right_pixels = get_lane_pixels(persp_transform, right_base)

        warped_with_lane_lines, left_curv, right_curv, dist_from_center = draw_lane_lines(image, left_pixels,
                                                                                          right_pixels, left_base,
                                                                                          right_base)

        lane_lines = transform_perspective(image=warped_with_lane_lines, M=M_inv)

        final = weighted_img(lane_lines, image)

        cv2.putText(final, "Lane Curvature: " + str(left_curv) + " (m)", (100, 100), cv2.FONT_HERSHEY_SIMPLEX, 2,
                    (255, 255, 255))
        cv2.putText(final, "Distance from center: " + str(dist_from_center) + " (m)", (100, 150),
                    cv2.FONT_HERSHEY_SIMPLEX, 2, (255, 255, 255))

    except Exception as e:
        print(e)
        return image

    return final

An example of the final image can be found below:

# Pipeline for a video

# Discussion